M O D E M S

-----------

[part 19 from "The Winn Rosch HARDWARE  BIBLE"]

[Brady, New York, 1988                        ]

(C) Copyright  1991  by  Sergey Shulgin [SHS].  You can copy

this document for non-commercial use without changes only.

This document  was  printed  by  Sergey  Shulgin from Moscow

Institute of Physics and Technology,  Laser Center  lab.  If

you have  some  interest to this problem,  please call me by

modem or voice.

Coord:  Dolgoprudny,  Moscow region, Moscow Institute

of Physics and Technology, Institutsky per, 9

Laser Center lab.

Phone:  (095)408-79-88 from 10:00 till 19:00 by voice

(095)408-79-88 from 19:00 till 08:00 by modem

[connect to "Player's Dream" BBS and send message for me]

+-----------------------------------------------------------+

|        Welcome to "Player's Dream" BBS station!!!         |

|     [1st BBS Station of United Group Games in Russia]     |

|                                                           |

|   Operating time: 19:00-08:00 Mon.-Sat.                   |

|                   24h Sunday.                             |

|                                                           |

|   phone:(095)408-79-88                                    |

|                                                           |

|   modem:up to 9600 with MNP protocol                      |

|                                                           |

|   SysOp: Yaroslav Levashov, Sergey Shulgin                |

+-----------------------------------------------------------+

INTRODUCTION

------------

For Some  reason known to God and the infidels who figure

out the most profitable hardware packages to call  computers,

the modem  -  perhaps  the  most  desired  and  used computer

peripheral - remains an option in all but a  random  sampling

of portable computers.  The modem is the one computer feature

that lets  you  display  the  personality  of  your  personal

computer to  the  outside  world.  It  puts you in touch with

on-line databases,   remote   computer   systems,   far-flung

friends, and  even  those  who  are still flinging around the

country.

The purpose  and function of the modem seem almost absurd

in their simplicity.  The modem merely connects your computer

to the telephone line. The need for the extra device seems so

absurd because both computer and  telephone  use  obstensibly

the same  stuff  for  making and moving messages - electrical

signals. Were not  the  giant  corporations  specializing  in

computers and telephones(we won't name names) not such avowed

rivals, you might suspect that they were in cahoots to  foist

such a contrived accessory on the computer marketplace.

Step back and look at what a  modem  does,  however,  and

you'll gain  new  respect for the device.  In many ways,  the

modern modem is a miracle worker.  For instance,  the best of

today's modems  can  squeeze  more  than  a  dozen  data bits

through a cable where only one should  fit.  Even  the  least

expensive generic  modem  operates  like  a  specialized time

machine that  can  bridge  the  century-wide  chasm   cutting

between state-of-the-art  computers  and  stone-age telephone

technologies.

Far from   the   hatchlings   of   some   plot   by   the

Military-industrial complex -  or  that  even  more  sinister

force, the  telephone company - modems are a necessary bridge

between digital and analog signals.  The modern modem usually

does much  more than connect.  Most are boxes chocked full of

convenience features that can make using  them  fast, simple,

and automatic.  The  best of today's modems not only make and

monitor the connection but even improve  it.  They  dial  the

phone for  you,  even  remembering  the  number  you want and

trying again and again.  They listen in until they's sure  of

good contact,  and  only  then  let  you  transmit across the

telephone line.  Some even have built in circuits  to  detect

and correct  the inevitable errors that creep your electrical

conversations.

1. MODEM OPERATING PRINCIPLES

-----------------------------

A modem  is  a  signal  converter   that   mediates   the

communications between  a computer and the telephone network.

The very name "modem" indicates the role that it  plays.  The

term is a  foreshortening of the words MOdulator/DEModulator.

As a modulator,the modem converts the digital, direct current

pulses used  by  the  computer  system  into an analog signal

containing the   same   information,   a    process    called

"modulation".

1.1 Modulating and Demodulating

-------------------------------

Modulation is  necessary  because  the  telephone  system was

designed even before electronics was invented and solid-state

digital circuitry  lay almost a hundred years off.  The first

pained words out of Dr.Bell's speaking telegraph were  analog

electrical signals,  the  same  juice  that flows through the

receiver of your own telephone.  Although strictly  speaking,

digital communications are older - the conventional telegraph

predates the telephone by nearly three decades  (Samuel  F.B.

Morse wondered  what  God  had  wrought  in  1844)  - current

digital technology is only a recent phenomenon.

The telephone  system  was designed only to handle analog

signals because that's all that speaking  into  a  microphone

creates. Over  the  years,  the  telephone system has evolved

into an elaborate international network capable  of  handling

millions of these analog signals simultaneously and switching

them from one telephone set to another, anywhere in the world

and possibly   beyond.   Although   telephone  companies  are

increasingly using  digital  signals  to  move   trunk   line

communications between   switching  centers,  the  input  and

output ends  of  the  circuit  still  end   in   conventional

analog-based telephones.

Modulation, and hence modems, are necessary because these

analog telephone  connections will not allow digital,  direct

current signals to pass freely - or at  all.  The  modulation

process creates  analog  signals  that  code  all the digital

information of the computer original but can  be  transmitted

through the voice-only channels of the telephone system.

DEMODULATION reverses  the  modulation  process.  At  the

other end  of  the  connection,  the  modem  as a demodulator

receives that analog-coded signal and converts it back to its

original digital   form   while  preserving  its  information

content.

1.2 The Carrier

---------------

The actual processor of modulation superimposes one signal on

another. The modem as modulator starts its modulation process

by generating a constant signal which is called the "carrier"

because it  carries  or  bears  the  load  of  the modulating

information. In most systems,  the carrier is a  steady-state

signal of  constant  amplitude(strength)  and frequency,  and

coherent phase.

1.3 Modulation

--------------

The signal that's electrically  mixed  with  the  carrier  to

modify some  aspect  of  it  is  given  the  same name as the

process, "modulation".  Changes in the modulation result in a

change in  the carrier-and-modulation mix.  The change in the

modulation makes a corresponding change in  the  carrier  but

not necessarily  a  change in the same aspect of the carrier.

For instance,  in FM or frequency modulation, a change in the

"strength" of  the modulation is reflected as a change in the

"frequency" of the carrier.

Modulation brings  several benefits,  more than enough to

justify the  complication  of  combining   signals.   Because

electronic circuits  can  be tuned to accept the frequency of

one carrier  wave  and  reject  others,  multiple   modulated

signals can  be  sent through a single communications medium.

This principle  underlies   all   radio   communication   and

broadcasting. In   addition,   modulation   allows   digital,

direct-current-based information to be transmitted through  a

medium, like  the telephone system,  that otherwise could not

carry direct current signals.

In demodulation,  the  carrier  is  stripped away and the

encoded information  is  returned  to  its   original   form.

Although logically   just   the   complement  of  modulation,

demodulation usually involves entirely different circuits and

operating principles, which  adds  to  the  complexity of the

modem.

1.4 Short-Haul Modems

---------------------

Some so-called  modems  aren't  even  modems  at   all.   The

inexpensive "short-haul  modems"  advertised  involve minimal

circuitry, definitely not enough to modulate  and  demodulate

signals. So little, in fact, that often it is entirely hidden

inside the shell of a simple cable  connector.  All  that the

short-haul modem  does  is  convert  the  digital output of a

computer to another digital form that an better withstand the

rigors of a thousand feet of wire.

2. CHANNEL LIMITS

-----------------

Like all great works of art, the modem is constrainted to

work within the limits of its medium,  the telephone channel.

These limits are imposed by  the  characteristics  of  analog

communications and  the  communications  medium  that's  used

(primarily unshielded twisted-pair wire).

2.1 Signal Bandwidth

--------------------

All communications  channels  and  the  signals  that  travel

through them   have   a  characteristic  called  "bandwidth".

Bandwidth merely specifies a range of  frequencies  from  the

lowest to  the  highest  that  the  channel  can carry or are

present in the signal.

An unmodulated  carrier  wave  has  a  nominal  operating

frequency. For  example,  in  radio  broadcasting,  it's  the

number you  dial  in  when you tune in your favorite station.

Without modulation,  the carrier  wave  uses  only  that  one

frequency and has essentially zero bandwidth.

The modulation  that's  added  to  the  carrier  contains

information that  varies  at  some  rate.  Traditional analog

signal sources - music  or  voice  signals,  for  instance  -

contain a  near-random  mix  of  frequencies  between  20 and

20,000 Hz.  Although digital signals start off as  DC,  which

also has  no bandwidth,  every change in digital state adds a

frequency component.  The faster the states change - the more

information that's  squeezed  down  the  digital channel,  as

measured in its  bit  rate  (bits  per  second)  -  the  more

bandwidth the signal occupies.

2.2 Sidebands

-------------

In the  simplest  modulation  systems,  a  modulated  carrier

requires twice  the  bandwidth  of  the  modulation   signal.

Although this  doubling  sounds  anomalous,  its  the  direct

result of the combining  of  the  signals.  The  carrier  and

modulation mix  together  and result in "modulation products"

corresponding to the frequency of modulation both "added  to"

the carrier together with the frequency of the modulation and

"subtracted from" the carrier.  The  added  result  is  often

called the  "upper  sideband"  and  the  subtracted  result i

correspondingly called the "lower sideband".

2.3 Channel Bandwidth

---------------------

The bandwidth  of  a  communications  channel   defines   the

frequency limits  of  the  signals  that  it can carry.  This

channel bandwidth may be physically  limited  by  the  medium

used by  the channel or artificially limited by communication

standards. For example,  the bandwidth of radio transmissions

are limited  artificially,  by  law,  to allow more different

modulated carriers to share the air  waves  while  preventing

interference between them.

In wire-based  communications  channels,  bandwidth  is

often  limited  by  the  wires  themselves.  Certain physical

characteristics of wires cause  degradations  in  their  high

frequency  transmission abilities.  The "capacitance" between

conductors  in  a  cable  pair,  for  instance,  increasingly

degrades  signals as their frequencies rise to the point that

a high frequency signal might not be able  to  traverse  more

than a few centimeters of wire.  "Amplifiers" or "repeaters",

which boost signals so that they can travel longer distances,

often  cannot  handle  very  low  or  very  high frequencies,

imposing more limits.

Most telephone channels also have an artificial bandwidth

limitation imposed by  the  telephone  company.  To  get  the

greatest financial  potential  from  the  capacity  of  their

transmissions cables,  microwave  systems,  and   satellites,

telephone carriers regularly limit the bandwidth of telephone

signals. One reason bandwidth is  limited  is  so  that  many

separate telephone  conversations  can be stacked atop of one

another through multiplexing techniques,  allowing  a  single

pair of    wires    to   carry   hundreds   of   simultaneous

conversations. Although the effects of  bandwidth  limitation

are obvious  - that's why your phone doesn't sound as good as

your stereo - the telephone  company  multiplexing  equipment

works so  well  that  you  are  generally  unaware of all the

manipulations made on the voice signals as they are  squeezed

through wires.

2.4 Bandwidth Limitations

-------------------------

One of   the   consequences   of   telephone  company  signal

manipulations is a severe limitation in the bandwidth  of  an

ordinary telephone  channel.  Instead  of  the full frequency

range of a good-quality stereo system,  from 20 to 20,000 Hz,

a telephone  channel  will only allow frequencies between 300

and 3000 Hz to freely pass.  This very narrow bandwidth works

well for  telephone  because frequencies below 300 Hz contain

most of the power of  the  human  voice  but  little  of  its

intelligibility. Frequencies   above   3000  Hz  increase the

crispness of  the  sound  but  don't   add   appreciably   to

intelligibility.

While intelligibility is the primary concern  with  voice

communications (most   of   the   time),   data  transfer  is

principally oriented to bandwidth.  The comparatively  narrow

bandwidth of   the  standard  telephone  channel  limits  the

bandwidth of the modulated signal it can carry, which in turn

limits the amount of digital information that can be squeezed

down the phone line by a modem.

Try some simple math and you'll see the harsh constraints

faced by your modem's signals.  A telephone channel typically

has a useful bandwidth of about 2700 Hz (from 300 to 3000 Hz)

At most a carrier wave at exactly the center of the telephone

channel, 1650 Hz, and burdened by both sidebands, could carry

data that varies at a rate of 1650 Hz.  Such a  signal  would

fill the  entire  bandwidth  of the telephone channel without

allowing for a safely margin.

2.5 Safely Margin

-----------------

A safely margin is necessary, however, because the quality of

telephone lines   varies   greatly,  particularly  when  long

distance connections are involved.  Because poor  connections

can't handle  the nominal 300 to 3000 Hz telephone bandwidth,

it's ill-advised for a modem to try to take advantage of that

entire frequency  spread.  If  the connection is substandard,

when the data rate reaches  the  fringes  of  the  bandwidth,

errors are likely to crop in.

2.6 Duplex

----------

The usable bandwidth of a data communications channel through

a modem is also limited because most modem communications are

handled in   "duplex"   mode.   The  term  "duplex"  -  often

redundantly called "full-duplex" - describes the ability of a

communications channel  to  simultaneously handle two signal,

usually (but not necessarily) going in  opposite  directions.

Using these  two  channels,  a full duplex modem can send and

receive information at the same time.  Two carriers are  used

to simultaneously   transmit  and  receive  data.  Using  two

carriers, of course, halves the bandwidth available to each.

2.7 Half-Duplex

---------------

The alternative to duplex communications is "half-duplex". In

half-duplex only  one  signal  is  used,  and  to  carry on a

two-way conversation  a  modem  must  alternately  send   and

receive signals.  It  allows more of the channel bandwidth to

be put to use  but  in  practice  slows  data  communications

because a   modem  often  must  switch  between  sending  and

receiving modes after every block of data crawls through  the

channel.

2.8 Echoplex

------------

The term "duplex" is often,  and mistakenly, used to describe

"echoplex"  operation.  In  echoplex  mode,  a  modem sends a

character down the phone line,  and the distant modem returns

the same  character thereby echoing it.  The echoed character

is then displayed on the originating terminal as confirmation

the character was sent correctly.  Without echoplex, the host

computer usually writes the transmitted character directly to

its monitor screen.

2.9 Guard Bands

---------------

Duplex does more than cut the  bandwidth  available  to  each

channel in  half.  Separating  the  two  channels is a "guard

band", a width of unused frequencies that isolate the  active

channels and   prevent   confusion   between  their  separate

carriers. The safety margin  is,  in  effect,  also  a  guard

between the carriers and the varying limit of the bandwidth.

Once you add in the needs of duplex communication and the

guard bands,   the   practical   bandwidth  limit  for  modem

communications over real  telephone  channels  that  have  an

innate 2700  Hz  bandwidth  works out to about 2400 Hz.  That

leaves 1200 Hz for each of the two duplex channels.

3. MODEM MODULATION METHODS

---------------------------

For the job of making modulation, a modem has several methods

available to  it  much  as  AM  and  FM  radio  stations  use

different modulation   methods.   The   different   forms  of

modulation all  are  based  on  the  characteristics  of  the

carrier wave that can be changed to encode information.

Three of the primary carrier characteristics  that  might

be used for modulation are its amplitude,  its frequency, and

its phase.

3.1 Amplitude Modulation

------------------------

The amplitude is the strength of the signal or  the  loudness

of a  tone  carried  through the telephone wire.  Varying the

strength of the carrier in response to modulation to transmit

information is called "amplitude modulation".

One way that digital  information  could  be  coded  with

amplitude modulation  is  as  two  discrete  strengths of the

signal corresponding to the two digital states.  In fact, the

most rudimentary  form  of  amplitude  modulation - which has

earned the special name "Carrier Wave" or  "CW"  transmission

uses the  two  limits of carrier strength for its code:  full

power and zero power.  The loudness of a telephone signal  is

its most likely characteristics to vary,  however,  with both

changes in the telephone line and noise that might be  picked

up by  the line.  Consequently,  pure amplitude modulation is

not used by modems.

3.2 Phase Modulation

--------------------

Another state  of the carrier that can be altered  to  encode

information is   its  phase.  An  unmodulated  carrier  is  a

constant train of  identical  waves  that  follow  one  after

another precisely  in  step.  If  one  wave  were delayed for

exactly one wavelength,  it would fit exactly atop  the  next

one. The  peaks  and troughs of the train of waves flow by at

constant intervals.

By delaying   one  of  the  waves  without  altering  its

amplitude or frequency,  a detectable state change  called  a

"phase shift" is created. The onset of one wave is shifted in

time compared to those that preceded it.  Information can  be

coded as  "phase modulation" by assigning one amount of phase

shift from the constant carrier to a digital one and  another

to a digital zero. Although this form of modulation is useful

in modem communications, it is most often used in combination

with other modulation techniques.

3.3 Frequency Modulation

------------------------

The other   alternative   modulation   technique  alters  the

frequency of the carrier is response to the  modulation.  For

example, a  higher  amplitude  of modulation might be made to

cause the  carrier  to  shift  upward  in   frequency.   This

technique, called "frequency modulation", is commonly used in

radio broadcasting by familiar FM stations.

3.4 Frequency Shift Keying

--------------------------

In the most rudimentary digital form of frequency modulation,

a digital one would cause the carrier wave to change from one

frequency to another.  In other words,  one  frequency  would

signify a  digital  one  and  another  discrete  frequency  a

digital zero.  This form of modulation is  called  "frequency

shit keying"  or FSK because information is encoded in (think

of it being "keyed to") the shifting of frequency. The keying

part of  the  name  is  actually  left  over from the days of

telegraphy when  this  form  of  modulation  was   used   for

transmitting Morse code and the frequency shift came with the

banging of the telegraph key.  Frequency shift keying is used

in the    most    rudimentary    of   popular   modems,   the

once-ubiquitous 300 bit per second modem that operated  under

the Bell 103 standard.

3.5 Baud Rates

--------------

With such  modems  one  bit  of data causes one corresponding

change of frequency in the  carrier  wave.  Every  change  of

frequency or  state  carries  exactly one bit of information.

The unit of measurement used to describe the number of  state

changes taking  in place in one second is the "baud".  In the

particular case of the FSK modulation,  one change  of  state

per second   -   one  baud  -  conveys  exactly  one  bit  of

information per second.

Depending on   the   number   of   states   used  in  the

communication system,  a single transition - one baud  -  can

convey less  than  or  more than one bit of information.  For

example, several different frequencies of tones might be used

to code  information.  The  changing  from  one  frequency to

another would take place at one  baud,  yet  because  of  the

different possible changes that could be made,  more than one

bit of information could be coded by that transition.  Hence,

strictly speaking,  one  baud  is not the same as one bit per

second, although the terms are often,  and incorrectly,  used

interchangeably.

The number of bits that can be coded by  baud  varies  by

the inverse  logarithm  of  the  number  of  available states

)tones, voltage or phases).  Most 1200 bit per second  modems

operate at 600 baud with four different states available, and

most 2400 bit per second modems operate at 600 baud  with  16

different states.

(In case you're interested,  the term  "baud"  was  named

after J.M.E.  Baudot, a French telegraphy expert. His name is

also used to describe a 5-bit digital code used  in  teletype

systems).

3.6 FSK Modems

--------------

This 300  bit  per second rate using the simple FSK technique

requires a bandwidth of 600 Hz.  The two  300  baud  carriers

(which require  a 1200 Hz bandwidth,  two times 600 Hz) and a

wide guard band fit comfortably within the 2700 Hz limit.

Under the  Bell  103 standard,  which is used by most 300

bit second modems,  the two carrier frequencies are 1200  and

2200 Hz.  Space  modulation (logical zero) shifts the carrier

down by 150  Hz,  and  mark  modulation  pushes  the  carrier

frequency up by and equal amount.

Because the  FSK  modulation  technique   is   relatively

simple, 300  baud  modems are generally inexpensive.  Because

they don't push out to the  limits  of  available  bandwidth,

they are generally reliable even with marginal connections.

Using the same simple modulation technique and exploiting

more of  the 2700 Hz bandwidth of the typical telephone line,

modem speeds can be doubled to 600 baud.  Beyond  that  rate,

however, lies the immovable bandwidth roadblock.

4. MODEMS FASTER THAN 300 Bits Per Second

-----------------------------------------

A data communications rate of 300 bits per second is  slow  -

slower than  most  folks  can  read  text  flowing across the

screen. Even the slowest PC can absorb information  at  least

32 times  faster,  limited  by  the maximum serial port speed

that IBM supports.  Were long distance communications limited

to the 300 bit per second rate,  the only people who would be

happy would be the  shareholders  of  the  various  telephone

companies. Information  could,  at best,  crawl slowly across

the continent.

By combining several modulation techniques, modern modems

can achieve much higher data rates through  ordinary  dial-up

telephone lines.  Instead  of merely manipulating the carrier

one way.  they may  modify  two  (or  more)  aspects  of  the

constant wave.  For  instance,  today's most popular 1200 and

2400 bit  per  second  modems  combine  frequency  and  phase

modulation to achieve faster data flow.

4.1 Quadrature Modulation

-------------------------

These more complex forms of modulation add no extra bandwidth

(remember, that's a function of the  communications  channel)

but they  take advantage of the possibility of coding digital

data as changes between a variety of states  of  the  carrier

wave. For example, the carrier wave can be phase modulated so

that it assumes one of four states.

In the   "quadrature   modulation"   (a   form  of  phase

modulation) used by most 1200 bit  per  second  modems,  each

state of  the  carrier  differs  from the unmodulated carrier

wave by a phase angle of 0,90,180,  or 270  degrees  -  while

operating at 600 baud.

4.2 Group Coding

----------------

The four  different phase states are sufficient to encode the

four different patterns of two digital bits.  Each  baud  can

hold two bits of data,  thus, a quadrature-modulated 600-baud

modem can communicate at its  date  rate  of  1200  bits  per

second. This  bit-packing  is  the key to advanced modulation

techniques. Instead of dealing with data one bit at  a  time,

bits of  digital code are processed as groups.  Each group of

data bits is encoded as one particular state of the carrier.

The ultimate  speed  of  the  mode  is  determined by the

number of  states  that  are  available   for   coding.   The

relationship is not linear, however. As the number of bits in

the code increases by a given figure (and thus the  potential

speed of  the modulation technique rises by the same figure),

the number of states required increases to the  corresponding

power of two.  Twice as fast requires four states; four times

faster requires 16 states;  eight times as fast requires  256

states; and so on.  Data rates of 2400 bps can be achieved by

using an even more complex modulation that yields 16 discrete

states while still operating at 600 baud.  Each state encodes

one of the 16 different patterns of four  digital  bits.  One

blip on the  telephone  line  carries the information of four

bits going into the modem.

More complex  methods  of  modulation  allow  even higher

modem speeds-dial-up modems operating at 4800 bps and  beyond

are already   available.  Most  higher  speed  modems  -  for

example, today's 9600 bps products - get  an extra  boost  by

foregoing duplex  transmission  and alternate between sending

and receiving.

5. HIGH SPEED MODEMS

--------------------

Modems that operate at data rates in excess of 2400 bits  per

second are  generally  classed  as  "high speed modems".  The

distinction is as qualitative as it  is  quantitative:  Above

2400 bps, squeezing more information into the confines of the

telephone line  becomes  increasingly  difficult,   requiring

inventive modulation  techniques  quite  unlike those used at

lower rates.

According to  the  free  lunch principle,  this system of

seemingly getting  something  for   nothing   using   complex

modulation must  have a drawback.  With high speed modems the

problem is that the quality of  the  telephone  line  becomes

increasingly critical   as   the   data  rate  is  increased.

Moreover, as modem speeds get faster,  each phone  line  blip

carries more  information,  and  a single error soon can have

devastating effects.

5.1 Leased-Line Modems

----------------------

One way to coax higher speed from a modem is  to  forego  the

one part  of the connection that imposes the severe bandwidth

limitation - the telephone line.  Special high-grade circuits

can be  rented  from  telephone  companies to whisk data from

point to point at almost unbelievably high data  rates  (from

ten thousand  to  millions  of bits per second).  The special

lines are semi-permanently  installed  and  stretch  directly

from one  location  to  another,  never  allowed  to  venture

through the rigors of the telephone switching system. Because

these special  lines  are  leased  by  the  month  (or  other

period), they are called "leased lines" and the  modems  that

use them are termed "leased-line" or "dedicated-line" modems.

They usually lack  the  dialing  and  answering  features  of

dial-up modems, and are meant for continuous connections.

5.2 Dial-Up Modems

------------------

In contrast,  the  modems  that  you are likely most familiar

with - the ones that tie into the telephone switching  system

- are  distinguished  as  "dial-up"  modems.  They  face  the

constraints of the telephone system and must  be  capable  of

dealing with  its  special problems and shortcomings.However,

they are the  most  useful  because  they  can  reach  nearly

anyone, anywhere  -  as long as the modems at the two ends of

the call are compatible with one another.

5.3 Line Compensation

---------------------

Although a  long  distance  telephone  connection  may  sound

unchanging to  your ear,  its electrical characteristics vary

by the moment.  Everything from a wire swaying in the Wichita

wind to  the  phone company's automatic rerouting of the call

through Bangkok when the direct circuits fill up  can  change

the amplitude,  frequency, and phase response of the circuit.

The modem then faces two challenges - not to  interpret  such

changes as  data and to maintain the quality of the line to a

high enough standard  to  support  its  use  for  high  speed

transmission.

5.4 Switching Modems

--------------------

Perhaps the   biggest  limit  imposed  on  high  speed  modem

communications is the  use  of  full  duplex  communications.

Because a  complete  duplex  modem circuit is essentially two

complete channels,  each can have (at  most)  only  half  the

telephone line's bandwidth available to it. Most of the time,

however, communications go only in one direction.  You key in

commands to  a  remote  access  system,  and  only  after the

commands are received does the remote system respond with the

information that  you  seek.  While  one end is sending,  the

other end is more than likely to be completely idle.

To make  better  use of the available bandwidth so-called

"switching modems" are designed  to  make  use  of  the  full

bandwidth of  the telephone channel,  switching the direction

of the signal as each end of the line  needs  to  send  data.

Such modems  are  able  to  achieve  a  doubling of data rate

without adding any complexity to their modulation.  In remote

mainframe access  situations,  where the protocol of the call

fits the mold of the two ends of the connections taking turns

using the  phone  line,  switching  modems can give a genuine

boost to the cross-country throughput of a modem system.

5.5 Asymmetrical Modems

-----------------------

Switching doesn't  always  work,  however.  The   change   of

direction of communication isn't instantaneous. The modem has

no way of knowing when to switch other  than listening for  a

pause in  the  data  stream.  Some  delay to recognize such a

pause must be  built  into  the  system.  Further,  when  the

direction of  the  call  changes,  the modem may be called to

adjust for  line  differences  (the  characteristics   of   a

telephone connection  are  not  necessarily  the same in both

directions because the two directions  of  communication  may

take entirely   different   paths).  In  all,  switching  the

direction of the data movement can take a full second.

While a  second  pause  may not be burdensome when you're

simply sending characters and  watching  a  response  on  the

screen, it can be overwhelming when transferring a file. Most

file transfer protocols(for example,  XMODEM and Kermit)  are

designed to  send a small block of data to the remote system,

which then checks it for accuracy and finally sends  a  brief

return message  that  the data was received intact or that it

was bad.  A switching modem may require a full second or more

for each turn-around and confirmation. In that some protocols

use blocks only 256 or 512 bytes long between  confirmations,

sending a  file  amounts  to  the  classic  hurry-up-and-wait

syndrome. The modems blast a  block  across  the  line,  then

lolly around for a much longer period awaiting a confirmation.

In an  attempt  to  get  the   best   of   both   worlds,

"asymmetrical modems"   cut  the  waiting  by  maintaining  a

semblance of two-way duplex communications  while  optimizing

speed "in  one  direction only" by shoehorning in a low speed

(typically 300 bps) channel in addition  to  the  high  speed

one. As  with  a  switching  modem,  asymmetrical  modems can

flip-flop the direction of  the  high  speed  communications.

They rely  on  algorithms  to determine which way is the best

way. Typically,  the  high  speed   channel   is   used   for

transferring blocks  of  data while the confirmations trickle

back on the lower speed channel.

5.6 Fallback

------------

Most modems  use,  at   most,   two   carriers   for   duplex

communications. These  carriers are usually modulated to fill

the available bandwidth.  Sometimes,  however, the quality of

the telephone  line  is  not  sufficient  to  allow  reliable

communications over the full bandwidth expected by the modem.

In such  case,  most high speed modems incorporate "fallback"

abilities. When the top speed does not work,  they attempt to

communicate at   lower  speeds  that  are  less  critical  of

telephone line quality.  The pair of modems might  first  try

9600 bps and be unsuccessful;  they might then try 4800, then

2400, and so on until reliable communications are established.

5.7 Multiple-Carrier Modems

---------------------------

While most modems  rely  on  a  relatively  complex  form  of

modulation on  one  or two carriers to achieve high speed,  a

few (notably the Telebit Trailblazer) use instead  relatively

simple modulation  on  multiple  carriers.  One  of the chief

advantages of this system  used  by  these  "multiple-carrier

modems" comes  into  play  when  the quality of the telephone

connection deteriorates. Instead of dropping down to the next

incremental communications rate, generally cutting data speed

in half,  the multiple-carrier modems  just  stop  using  the

carriers in  the  doubtful  regions  of  the  bandwidth.  The

communication rate may fall off only a  few  percent  in  the

adjustment. (Of  course,  it could dip by as much as a normal

fallback modem as well).

5.8 Data Compression

--------------------

Although there's no way of increasing  the  number  of  bits

that can  cross  a telephone line beyond the capacity of the

channel, the  information  handling  ability  of  the  modem

circuit can be increased by making each bit more meaningful.

Many of the bits that are sent through the telecommunication

channel are  meaningless  or  redundant  -  they  convey  no

additional information. By eliminating those worthless bits,

the information content  of the data stream is more intense,

and each bit is more meaningful. The process of  paring  the

bits is called "data compression".

The effectiveness of compression varies with the tye  of

data that's being transmitted.One of the most prevalent data

compression   schemes   encodes   repetitive    data - eight

recurrences of  the  same  byte  value might be coded as two

bytes, one signifying the value,  and the second the  number

of repetitions.This form of compression is most effective on

graphics, which often have many  blocks  of repeating  text.

Other compression  methods  may strip out start,  stop,  and

parity bits.  Modem  manufacturers  often  claim that  their

proprietary data compression methods might reduce the number

of bits  that  need  to  be  transferred  by   50   percent,

effectively doubling communications speed.

5.9 Error-Checking

------------------

Because all higher speed modems operate closer to the limits

of the telephone channel,  they are naturally more prone  to

data errors.  To better cope with such problems,  nearly all

high speed modems have their own  built-in  "error-checking"

methods. These  work like communications protocol - grouping

bytes into blocks and sending cyclical  redundancy  checking

information. They   differ   from   the  protocols  used  by

communications software in that they are implemented  in the

hardware instead  of  your  computer's software.  That means

that they don't load down your computer when  it's straining

at the limits of its serial ports.

It can also mean that software protocols  are  redundant

and a  waste  of  time.  As  mentioned before in the case of

switching modems,  using  a  software-based   communications

protocol can   be  counterproductive  with  many  high-speed

modems, slowing the transfer rate to a crawl. Most makers of

modems using  built-in  error-checking  will  advise against

using such software protocols.

6. MODEM CONTROL

----------------

Besides its  basic  purpose  of converting digital data into

modulated audio signals,  the modem is often called upon  to

handle other chores of convenience.  For example,  it may be

called upon to automatically dial or  answer  the  phone  or

report the  condition of the telephone line.  These features

of the modem must be able to be controlled by your computer,

and the  modem  must  be  able to signal your computer about

what it does and what it finds out.

6.1 Dual Modes

--------------

Most modems operate alternately in  one  of  two  modes.  In

"command mode",   the   modem   receives   and  carries  out

instructions sent  by  your  computer.  In   "communications

mode", it  operates as transparently as a modem can,  merely

converting data.

Changing modes  is  mostly  a  matter of sending control

characters to the modem. The characters can only be received

and processed in command mode.  In communication mode,  they

would be passed along down the telephone line.

6.2 Hayes Command Set

---------------------

Today, most modems use a standardized  set  of  instructions

called the  "Hayes  command  set",  after  Hayes  the  modem

manufacturer (which was,  in turn, named after Dennis Hayes,

its founder).  For  the  most  part,  the  Hayes command set

comprises several dozen modem instructions that begin with a

two character  sequence  called  "attention character".  The

sequence is almost mnemonic - the letters AT,  which must be

capitals. Other  characters  specifying  the command  follow

the attention character.  Because the AT is part  of  nearly

every command, the  Hayes command set is also termed the "AT

command set",  most often by Hayes' competitors  that  don't

want to   give   the   competition   credit.  A  modem  that

understands the Hayes command set (or the AT command set) is

said to be "Hayes-compatible". (The basic Hayes command  set

is listed in Appendix A of this document).

Most AT  commands  follow  the attention characters with

one letter that specifies the  family  of  the  command  and

another character  that indicates the nature of the command.

For example,  H stands for Hook.  H0 means put the phone "on

the hook"  or  hang  up.  H1 indicates that the modem should

take the phone off the hook,  that is,  make a connection to

the line.

Several commands and their modifiers can be  combined on

a single  line  after  an  initial  attention  command.  For

example, to command a Hayes  or  Hayes-compatible  modem  to

dial information on a tone-dialing line, the proper sequence

of commands  would  read:  ATDT15511212.  The  AT   is   the

attention signal,  D  is  the Dial command,  the T tells the

modem to use tones for dialing,  and  the  15511212  is  the

number of the telephone company information service.

All AT commands must be followed by a  carriage  return.

The modem waits for the carriage return as a signal that the

computer has sent the complete command and  that  the  modem

should start processing it.

At first   it   would   appear   that   shifting    into

communications mode would be a one-way street for the modem,

particularly were it only able  to  receive  instruction  in

command mode.  Fortunately, the Hayes command set allows the

modem to react to  exactly  one  command  in  communications

mode, a  command  that  instructs  the  modem  to  break off

communications and shift back to command mode.

The tricky  part  of designing such a command it that it

must be a character sequence that will never appear  in  the

data that  the  modem is supposed to be communicating across

the telephone line.  Although it's impossible  to  guarantee

that any  command  sequence  will never appear in the normal

progress of communications,  the command in the Hayes set is

specifically designed  to  be  statistically unlikely.  This

command simply consists of a string of three "plus  signs" -

that is,  +++.  To make the command stand out from data, the

Hayes command set also specifies that the three  plus  signs

be isolated  from  any  other  characters  by  at  least one

second, before and after.  Such a pause  followed  by  three

specific characters,  followed  by  a  pause  should   never

occur (well,  almost) except when  the  command  is   really

meant.

6.3 Extended Hayes Command Set

------------------------------

At the time the Hayes command set was developed,  modems had

relatively few  special  features.  As  modems  became  more

sophisticated, they  became  more  loaded with abilities and

features. The original Hayes command set had to be  extended

to handle    all   the   possibilities.   Note   that   many

Hayes-compatible modems recognize only the  original command

set. All  of  their  features  - if they have them - may not

work with software that expects the extended Hayes set.

6.4 S-Registers

---------------

The extensions to the original  Hayes  command  set  include

sufficient new  functions  that  the command language  would

become ungainly and confusing.  After all, there are only 26

letters in  the  alphabet  that might be used for one-letter

commands. Hayes added the facility of a special  register or

memory area  called  the "S-register" inside its modems that

allows the settings of the modem's operating  parameters. By

setting the value contained by the S-register, a  variety of

modem functions can be controlled.  (S-register settings are

shown in Appendix B of this document)

6.5 Response Codes

------------------

Commands sent to a Hayes-compatible modem by their very name

and nature are one-way.  Absent some means of  confirmation,

you would  never  know  whether  the modem actually received

your command,  let alone acted upon it.  Moreover,  you also

need some  means for the modem to tell you what it discovers

about your connection to the telephone  line.  For  example,

the modem  needs to signal you when it detects another modem

at the end of the line - and when that connections is broken.

Part of  the  Hayes command set is a series of "response

codes" which serve that feedback function.  When  the  modem

needs to  tell  you something,  it sends back - via the same

connection used to send data between your computer and modem

- code numbers or words to appraise you of the situation. In

the Hayes scheme of things,  you can set the modem  to  send

simple "numeric" codes, consisting solely of codes(which you

can then look up in your modem manual,  if you have one)  or

"verbose" responses,  which may be one or more words long in

something close to everyday English.

Typical responses include "OK" to signify that a command

has been received and acted upon, "CONNECT 1200" to indicate

that you've linked with a 1200 bit  per  second  modem,  and

"RING" to  show  that  the  phone  at  the  other end of the

connection is ringing.  (Hayes response codes are listed  in

Table below).

Note that because the  response  codes  flow  from  your

modem to  your  computer as part of the regular data stream,

you may accidentally confuse them with text  being  received

from the far end of your connection.

+---------+----------------+-------------------------------+

| Numeric |    Verbose     |                               |

|  code   |     vcode      |         Definition            |

+---------+----------------+-------------------------------+

|    0    | OK             | Command executed without error|

|    1    | CONNECT        | Connection established(300bps)|

|    2    | RING           | Phone is ringing              |

|    3    | NO CARRIER     | Carrier lost or never detected|

|    4    | ERROR          | Error in command line or line |

|         |                | too long                      |

|    5    | CONNECT 1200   | Connection established at 1200|

|    6    | NO DIALTONE    | Dialtone not detected in      |

|         |                | waiting period                |

|    7    | BUSY           | Modem detected a busy signal  |

|    8    | NO ANSWER      | No silenced detected while    |

|         |                | waiting for a quiet answer    |

|   10    | CONNECT 2400   | Connection established at 2400|

+---------+----------------+-------------------------------+

7. MODEM FEATURES

-----------------

The broad  term  "features"  describes  various subtle - and

some not-sosubtle - ways in which  modems  differ  from  one

another. For  the  most part,  the features of a modem taken

together determine how easily and conveniently you  can  put

it to work. A no-frills modem, for example, may require that

you spin the dial of your phone with your  index  finger  or

answer incoming  calls  before  turning  them  over  to your

computer system when you hear the carrier tone of  the modem

at the other end of the line. Many people are willing to put

up with such petty inconveniences to save on the price  of a

modem.

Although none  of  the  tasks  that   features-deficient

modems foist  upon you will tax your mind or constitution, a

no-frills modem  short-changes  the  capabilities  of   your

computer. With  a full-featured modem,  your PC can dial the

phone faster and with fewer errors and can handle  the chore

automatically when  you're  not  around.  Or with the latest

memory-resident communication software,  your  PC  can  dial

the full-featured  modem and collect your messages while you

are in the midst of browbeating data into shape with another

program.

Actually, nearly every modem made today -  including the

lowest budget models made in obscure foreign lands - has all

the standard features that you might normally want. Once you

start integrating features into circuit chips,  adding a few

more features is not  an  arduous  process.  The  only  time

you're likely  to  run  into  a  modem  deficient in today's

convenience features is when you try to  make  do  with  one

manufactured to   yesterday's  standards  -  the  modem  you

inherit from some corporate higher-up,  one  that  you  find

lying face-down  in  the  gutter and you nurse back to life,

one that you buy used from a shady-looking  character  in  a

trench coat on a deserted street corner.

Of the various features of a state-of-the-art modem, the

ones you  should  expect  in  any  new  product that you buy

include:

7.1 Auto-Answer

---------------

An auto-answer  modem  is  capable  of  detecting   incoming

ringing voltage(the low-frequency,  high-voltage signal that

makes the  bell  on  a  telephone  ring)  and  seizing   the

telephone line  as if it had been answered by a person. Upon

seizing the phone line, the auto-answer modem sends a signal

to its  host computer to the effect that it has answered the

phone. The computer then can interact with the caller.

An auto-answer  modem  allows  you or other to call into

your computer system without anyone being present  to answer

the telephone and make the connection to your computer.

7.2 Auto-Dial

-------------

An auto-dial  modem  is  capable of generating pulse-dial or

DTMF (dual-tone modulated frequency or  touch-tone)  dialing

signals independent of a telephone set.

An auto-dial modem can dial the telephone under computer

command, for  example,  after  hours  when you're asleep and

phone rates are low.  Without auto-dial,  you would have  to

dial the  phone  yourself,  listen  for  the  screech of the

far-end modem's answer, plug in your modem, and finally hang

up the phone.

7.3 Automatic Speed Sensing

---------------------------

Before a connection is made,  you may have no way of knowing

at what speed a distant modem will  be  operating.  Most  of

today's modems can automatically adjust for the speed of the

distant modem - if it is within the speed range that  can be

handled. High  speed  modems  usually  negotiate the highest

possible shared  speed  to  operate  at  using   proprietary

protocols.

Many modems also attempt to adjust to the speed at which

you send  them data - again if is within the range of speeds

that the modem can handle.  The attention code of the  Hayes

command set  conveys  enough data that a modem can lock into

the data and appropriately match its operating speed to that

of the information flow.

7.4 CCITT Compatibility

-----------------------

A branch  of  the United Nations,  the CCITT,  which roughly

translated from  the  original  French  means   "Cooperative

Committee for  International  Telephony  and Telegraphy" has

created a number of communications standards that  have  won

great worldwide acceptance. Many of these standards apply to

modems. For   example,   many   modems   boast   of   "CCITT

compatibility" with   the  v.22  standard,  which  describes

operation at a data rate of 2400 bits per second.  At higher

speeds other  CCITT standards are  gaining  popularity, such

as v.32 for 9600 bit per second modems.

In theory,  the  adoption  of the CCITT standard is good

news for people who want to communicate overseas  (where the

Bell standards  may be illegal).  The principal value of the

CCITT standards is,  however,  that many  manufacturers  are

taking them  to  heart  and  designing  products  to  match,

increasing compatibility   and    eliminating    marketplace

confusion.

7.5 Acoustic Couplers

---------------------

Really vintage   modems  made  no  electrical  contact  with

telephone lines at all.  That's because  years  ago  hooking

your modem  directly to the phone line was neither practical

or legal.  It wasn't practical before the now-common modular

telephone plug-and-jack  arrangement  allowed anyone to plug

in telephone equipment  without  fear  of  embarrassment  or

electrocution. It  wasn't  legal  because  telephone company

regulations dating long before the AT&T  telephone  monopoly

was split  up did not permit individuals to directly connect

modems to their telephone lines.

Instead of  electrical connections,  vintage modems sent

their signals to telephones as sound waves.  A device called

an "acoustic  coupler"  was  used  to  convert the tone-like

analog signals made by the modem into sounds which  are then

picked  up  the   microphone  in  the  telephone handset and

passed through the telephone  network  again  as  electrical

signals. To  make the sound connection a two-way street, the

acoustic coupler also incorporated a microphone to  pick  up

the squawks  emanating  from the earpiece loudspeaker of the

telephone handset, convert them into electrical signals, and

supply them to the modem for demodulation.

Acoustic couplers  can  take  many   forms.   In   early

equipment, the  acoustic coupler was integral to the modem -

a special cradle in which you  lay  the  telephone  handset.

Today you're  more  likely  to  see  couplers  made from two

rubber cups designed to engulf the mouthpiece  and  earpiece

of a telephone handset. This latter form of acoustic coupler

persists because it allows modems to  be  readily  connected

and disconnected  from  non-modular  telephones - those that

you cannot  unplug  to  directly  attach   a   modem.   This

connectability is  especially important for roving computers

that may be called upon to tie their  internal  modems  into

nonmodular pay station and hotel room telephones.

7.6 Direct-Connect Modems

-------------------------

Modems that  directly  plug into the electrical wires of the

telephone system    are     called,     quite     logically,

"direct-connect" modems. Almost in tribute to the acceptance

of the modular telephone wiring system,  nearly every  modem

that you can buy today is direct-connect.

7.7 Asynchronous Modems

-----------------------

Almost any  modem  that  you buy for normal use with your PC

will feature "asynchronous transmission".  This odd-sounding

term describes  a  method  of exchanging information between

two different  computer  systems  that  operate   completely

independently and do not share any timing information.

Normally, the time at which a pulse occurs  in  relation

to the  ticking  of a computer's system clock determines the

meaning of a bit in a digital signal, and the pulses must be

synchronized to   the   clock   for   proper  operation.  In

asynchronous transmissions,  however, the digital pulses are

not locked  to the system clock of either computer. Instead,

the meaning of each bit of a digital word is defined  by its

position in  reference  to  the  clearly (and unambiguously)

defined start bit.  Because the timing is  set  within  each

word in isolation, each word of asynchronous signal is self-

contained and essentially independent of any  time relations

beyond its self-defined bounds.

The signals of modems that use the telephone  system are

generally asynchronous  because  it  is  more  expensive and

difficult to  synchronize  signals  through  the   telephone

system through  which  signals  may  be rerouted at any time

without any warning.

7.8 Synchronous Modems

----------------------

Most dedicated-line  modems  use  a  special   communication

technique often  used  among  mainframes called "synchronous

transmission". In this method of  transmitting  data  across

phone lines, the two ends of the channel share a common time

base and the communicating modems  operate  continuously  at

substantially the   same   frequency   and  are  continually

maintained in the connection  and  adjust  for  the  circuit

conditions. Higher  speed  modem  - 2400 bits per second and

beyond - often use synchronous transmissions.

In synchronous  transmissions  the  timing  of  each bit

independently is vital,  but framing bits  (start  and  stop

bits) are    unnecessary,   which   makes   this   form   of

communication a bit - actually two or three bits  -  faster.

One problem  in  using it  is that before information can be

exchanged, not just the two ends of the connection  must  be

synchronized, but  also  the  link  between  the  modem  and

computer must be synchronized.  Autodialing features usually

won't work  in synchronous mode because without a connection

being made there's nothing  to  synchronize  to  -  and  the

connection cannot be made without dialing.

7.9 Autosynchronous Modems

--------------------------

Hayes solved    the    dialing   problem   for   synchronous

communications by adding  an  "autosynchronous"  feature  to

their newest  higher speed modems.  This special mode allows

the connection   between   PC   and   modem    to    operate

asynchronously. The  modem  translates  those  signals  into

synchronous mode before  sending  them  down  the  telephone

line. It also works the other way and translates synchronous

signals from the far end of the line into  asynchronous  for

sharing with the host computer. The autosynchronous features

can help PCs talk to mainframe and other computers  that use

synchronous communication as easily as they communicate with

other PCs.

7.10 Modem Packaging

--------------------

Perhaps the biggest choice you have in buying a new modem is

whether in installs inside your PC as an "internal modem" or

connects outside your PC through a  cable  as  an  "external

modem". Internal  modems  are like any other expansion cards

that plug into a vacant slot inside your PC. External modems

are additional boxes to find a place for on your desk.

In many cases When you have  to  choose  between  actual

products, physical  appeal  may  be  the best guiding factor

because exactly the same circuitry is often available in the

different packages.

There are a few practical  reasons  for  preferring  one

style of modem packaging over another. External modems offer

the advantage of portability.  You can  move  your  external

modem between  different  computer  systems (even those that

are not IBM compatible) simply by pulling the  plug.  Moving

an internal  modem  requires popping the lid off your PC and

the recipient and all the folderol that follows.

Additionally, internal   modems   can  restrict  you  to

certain computer systems.  Some internal modems are built as

full-length expansion   cards,  which  means  you  can  only

install them  in  full-size  PCs,  XTs,  ATs,  and  hardware

compatibles. You'll   need   a   different   modem   for   a

foreshortened computer (like the Tandy 1000).  When you make

the move  from PC to PS/2 architecture,  you'll also have to

shell out the cash for a new internal  modem.  Most  laptops

and the   PCjr  can  use  only  internal  modems  that  were

specifically designed for their proprietary expansion buses.

If you  have  a PC with its original,  minimal 63.5-watt

power supply,  adding  an  internal  modem   -   an   older,

full-length modem  card in particular - may limit the number

of other expansion  boards  that  you  can  plug  into  your

system. Such  modems  are  notoriously  power-hungry and may

leave few watts for other cards,  such as hard disks and EMS

boards.

On the other hand,  internal modems tend  to  be  a  few

dollars cheaper  than  external  models because the internal

units don't need extra packaging or power supplies (although

they need some extra signal circuitry).  You can also forego

the cost of a serial cable, which might cost you $30 or more

from a local dealer.  With an internal modem, you don't have

to deal with a tangle of cords,  plugs or transformers vying

for the  few holes in your wall outlet,  extra boxes on your

desk, or another thing to  switch  off  when  you  put  your

system to sleep at night.

7.11 Port Assignments

---------------------

Other than  matters  of  power  supply,  the  impact on your

system resources will be the same no  matter  the  style  of

modem you  choose.  While  external  modems require a serial

port and cable,  internal modems also require the use  of  a

serial port  address - which means you still lose the use of

that address by a serial port, COM1 or COM2 (or COM3 or COM4

in PS/2s  and the latest DOS versions).  If you use versions

of DOS before 3.3,  you'll only be able to  add  one  serial

port in  addition  to  the port or address used by either an

external or internal modem.  While some internal modems  can

have their addresses to be set as COM3 or COM4,  you must be

sure that the communications software that  you  choose  can

control the ports beyond COM2.

If there is any  general  rule,  it's  that  you  should

choose an external modem for its flexibility and its ability

to move to new and different  computer  systems;  choose  an

internal modem for its neatness and lower overall cost.

8. OTHER MODEM CONSIDERATIONS

-----------------------------

Selecting one modem from the hundreds of  products currently

available is  no  small task.  However,  it can be made more

tractable by  making  four  separate  judgments  about  each

particular modem's performance, compatibility, features, and

price.

Over a   perfect   telephone  line,  nearly  all  modems

function perfectly  -  without  errors.   However,   perfect

telephone lines are impossible to find,  and even getting an

acceptable one  nowadays  seems  to   require   bribing   an

operator. The  performance differences between modems appear

as line quality goes down.  Better modems are better able to

cope with bad connections. They work with worse circuits and

can pull data through with fewer errors.

One of  the critical parameters of the telephone line is

the amount of noise it contains in relation to  the strength

of the  signal  it  carries.  This  relationship  is usually

termed the signal-to-noise ratio of  the  line.  The  higher

this ratio is, the better the connection.

The signal-to-noise ratio is often expressed in decibels

(one tenth  of  a measurement unit called the Bel,  which is

named after Alexander Graham Bell,  by the way) which form a

logarithmic measurement scheme - that is,  a signal-to-noise

ratio that is twice as good will only appear  three decibels

better.

The compatibility of a modem refers to software  and not

its hardware connection.  Controlling all the features of an

advanced dial-up modem requires that commands be sent  to it

from the  computer  that it is connected to.  These commands

are usually sent invisibly  by  the  communications  program

that is   actually   in   control.  More  compatible  modems

recognize the commands of a wider range of software packages.

Obviously, to  be  useful  communications  software must

know the  commands   that   the   modem   recognizes.   Some

communications programs  let  you  define the modem commands

yourself (usually during the installation  process)  and  in

theory will    work    with   any   modem.   However,   most

communications programs are designed to accept a one or more

standard sets of commands.

In dial-up modems, no commands set has received official

sanction as  the  one and only standard.  Most modems today,

however, follow a de facto standard, the Hayes or AT command

set. Originally  developed by Hayes Microsystems and used in

the company's line of Smartmodems,  the  popularity  of  the

hardware led  to  many  software companies incorporating the

commands in their products.  Newer modems  were  adapted  to

accept the existing software,  resulting in an acceptance of

the standard.  A modem that recognizes the Hayes command set

will therefore  work  with  the  widest variety of software.

Hayes-compatible modems are thus more versatile.

Note, however, that the Hayes standard is not immutable.

As new modem features and capabilities  are  developed,  the

commands set  becomes richer.  Moreover,  varying degrees of

Hayes compatibility exist.  Some modems only  recognize  the

most rudimentary  of  commands,  for example,  using ATDT to

initial the dialing sequence.  Other more elaborately  mimic

the operation  of  Hayes  products  and incorporate the same

registers as used by Smartmodems, which permit, for example,

setting the  number  of  rings  required  before  the  modem

answers.

Unless you  have masochistic tendencies or software that

is specially designed for another  modem  command  set,  the

safe bet  is selecting a modem that's as Hayes compatible as

possible.

The price  difference  between  different  manufacturers

dwarfs the  difference  between  the  internal  an  external

modems of  any  given  manufacturer.  Exactly  how  much you

should spend depends on what you're  looking  for  and  what

you're willing to settle for.  As with any other PC product,

you should carefully consider every  aspect  of  your  modem

purchase before  making  your decision - select the one that

you're absolutely sure you want, then settle for the one you

can afford.

Unlike other common peripherals,  modems often  are  not

plug-and-play devices, external modems particularly. Perhaps

that's to be expected because they plug into  ornery  serial

ports.

8.1 Modem Cabling

-----------------

The easiest  part  of  installing  a modem is hooking up the

cable. MOdems connected to the serial ports used by  PCs and

PS/2s use  "straight-through"  cables.  Only AT-style serial

ports with nine-pin connectors  require  adapters  to  match

them to most modems.

The problems begin with software.  In  the  interactions

various communications  packages make with modems,  a number

of serial port control lines are  brought  into  play.  Some

communications programs,  such  as  PC-Talk III make minimal

use of the indications modems provide.  Others monitor every

connection. Consequently,  the  number of control lines that

must be connected - and thus the number of wires  that  must

be available  in  the  cable  that  links your modem to your

computer - depends on the communications software  you  plan

to use.  In some situations the minimal triad - pins 2,3 and

7 - will suffice. Other programs require the full complement

of ten connections. The moral to be drawn from this story is

that should you not know the type of cable required  by your

modem, use  a straight-through serial cable equipped with at

least ten conductors.

8.2 Modem Switch Settings

-------------------------

Modems themselves can be programmed to treat  their  various

connections in   different   ways  to  match  the  needs  of

software. For example,  some programs require that the modem

keep them  abreast  of  the  connection  through the Carrier

Detect signal.  Other  programs  couldn't  care  less  about

carrier detect  but carefully scrutinize Data Set Ready.  To

accommodate the range of communications  applications,  most

modems have  setup  switches  that determine the handling of

their control lines.  In one position,  a switch  may  force

Carrier Detect  to  stay  on continually,  for example.  The

other setting might cause the status of  Carrier  Detect  to

follow the state of the modem's conversations.

These switches   take   two   forms,   mechanical    and

electrical. Mechanical  switches  are  generally  of the DIP

variety. In  the  prototypical  modem,  the  original  Hayes

Smartmodem 1200,  these switches are hidden behind the front

panel of the modem.  (To get to them,  carefully pry up  the

trailing ears  of  the  sides of the bezel,  first one side,

then the other of the black front panel of the  modem.  Then

pull it forward and off).

Most commercial  modems  that  use  DIP   switches   are

patterned after  the Hayes Smartmodem 1200.  (Its DIP switch

settings are shown in Appendix C of this document).

The other  kind of switch is electrical,  exemplified by

the Smartmodem 2400. Made from EEPROM memory, these switches

are set by sending commands to the modem from your computer.

Because of their EEPROM nature,  they retain their  settings

even when the modem is turned off or unplugged.

Other modems may follow this pattern exactly or  may use

another memory  technology - for example,  battery backed-up

dynamic RAM. A few don't make any effort toward removing the

volatility. Such  modems  require  you  to  reprogram  their

settings every time you turn them on.  While  you  can't  do

much to  make  modem memory non-volatile when it's not,  you

can make life easier using disk memory. Simply add the modem

settings you  wish to enforce to the setup strings that many

communications software packages send to  the  modem  before

they begin to make a connection.

(The setup commands for the industry-standard Smartmodem

2400 are shown in Appendix D of this document).

P.S: Appendices A-D will be available later in our "Player's

Dream" BBS station. Make your orders, gentlemen!